CPE301 – SPRING 2019

# Design Assignment 3B

Student Name: Allis Hierholzer

Student #: 2000160356

Student Email: hierholz@unlv.nevada.edu

Primary Github address: https://github.com/acexhp/submission_da.git

Directory: Repository/cpe301/DesignAssignment/DA3B

Task:

The goal of the assignment is to modify the above codes to do the following:

1. Write a C AVR program that will monitor the LM34/35 connected to an Analog pin (PC5) to display the temperature in F on the serial terminal every 1 sec. Use a timer with interrupt for the 1 sec delay. Use a FTDI chip for serial to USB conversion.

2. Use the ATMEL Studio Data Visualizer or any Charting program to display the values in time.n.

Submission:

The following are required for successful completion of the design assignment:

a. AVR C code that has been compiled and working.

b. The C code should be well documented with explanation of every instruction.

c. A word document that contains the flow chart of the assembly code along with the snapshots of the schematics, components connected on the breadboard and screen shoots.

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- Breadboard

- Wires

- USB Cables

- ATMEGA328P XPLAINED MINI

- ATMEL STUDIO 7.0

- FTDI chip

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* DA3B.c

*/

#define F_CPU 16000000UL

#define BAUD_RATE 9600

#include <avr/interrupt.h>

#include <avr/io.h>

#include <util/delay.h>

int count_ovfl=0;				//counter for overflow

void usart_init ();

void usart_send (unsigned char ch);

int main (void)

{

usart_init ();

/** Setup and enable ADC **/

ADMUX = (0<<REFS1)|				// Reference Selection Bits

(1<<REFS0)|					// AVcc - external cap at AREF

(0<<ADLAR)|					// ADC Left Adjust Result

(1<<MUX2)|					// Analog Channel Selection Bits

(0<<MUX1)|					// ADC5 (PC5 PIN27)

(1<<MUX0);

ADCSRA = (1<<ADEN)|				// ADC ENable

(0<<ADSC)|					// ADC Start Conversion

(0<<ADATE)|					// ADC Auto Trigger Enable

(0<<ADIF)|					// ADC Interrupt Flag

(0<<ADIE)|					// ADC Interrupt Enable

(1<<ADPS2)|					// ADC Prescaler Select Bits

(0<<ADPS1)|

(1<<ADPS0);

TIMSK0 |= (1<<TOIE0);

TCNT0 = 0;					// setting initial value for counter

sei();						// enable global interrupts

TCCR0B |=(1<<CS02);				// setting prescaler to 256

while (1)

{

ADCSRA|=(1<<ADSC);			//start conversion

while((ADCSRA&(1<<ADIF))==0);	//wait for conversion to finish

ADCSRA |= (1<<ADIF);

if (count_ovfl==244)			// data is updated if overflow is 244 times

{

int a = ADCL;

a = a | (ADCH<<8);

a = (a/1024.0) * 5000/10;

usart_send((a/100)+'0');

a = a % 100;

usart_send((a/10)+'0');

a = a % 10;

usart_send((a)+'0');

usart_send('\r');

count_ovfl=0;

}

}

return 0;

}

ISR(TIMER0_OVF_vect)

{

while (!(TIFR0 & 0X01)==0);

count_ovfl++;					//increasing overflow counter

TCNT0=0X00;					//resetting counter to zero

TIFR0=0X01;					// reset the overflow flag

}

void usart_init (void)

{

UCSR0B = (1<<TXEN0);

UCSR0C = (1<< UCSZ01)|(1<<UCSZ00);

UBRR0L = F_CPU/16/BAUD_RATE-1;

}

void usart_send (unsigned char ch)

{

while (! (UCSR0A & (1<<UDRE0))); 		//wait until UDR0 is empty

UDR0 = ch;					//transmit ch

}

void usart_print(char* str)

{

int i = 0;

while(str[i] != 0)

usart_send(str[i]);

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/TrDcCMflFdY

- GITHUB LINK OF THIS DA

https://github.com/acexhp/submission_da.git

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Allis Hierholzer