CPE301 – SPRING 2019

# Design Assignment 4A

Student Name: Allis Hierholzer

Student #: 2000160356

Student Email: hierholz@unlv.nevada.edu

Primary Github address: https://github.com/acexhp/submission_da.git

Directory: Repository/cpe301/DesignAssignment/DA4A

Task:

The goal of the assignment is to modify the above codes to do the following:

1. Write an AVR C program to control the speed of the DC Motor using a potentiometer connected to PC0. Use an interrupt on a button (PC1/2/3) to stop and start the motor at each click. The minimum speed of the motor should be 0 when pot is minimum and maximum should be 95% of PWM value.

Submission:

The following are required for successful completion of the design assignment:

a. AVR C code that has been compiled and working.

b. The C code should be well documented with explanation of every instruction.

c. A word document that contains the flow chart of the assembly code along with the snapshots of the schematics, components connected on the breadboard and screenshots.

- COMPONENTS LIST

- Breadboard

- Power Supply

- Wires

- USB Cables

- ATMEGA328P XPLAINED MINI

- ATMEL STUDIO 7.0

- Multifunction Shield

- TB6612FNG Driver IC for DC Motor

- DC Motor

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* DA4A.c

*/

#define F_CPU 16000000UL		//clock runs at 16 MHz

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

#include <stdio.h>

#define MTR_1 5

#define SWITCH (PINC&(1<<3))    	// switch on PINC3

int DCMotor=0;			//variable for on/off motor

int main(void)

{

PORTC |=(1<<2);		//pull-up resistor

OCR0A = 200;			//setting top = 200, frequency

//sets timer/counter control register, mode 7, fast PWM and prescaler is 256

TCCR0A |= (1<<COM0A1)|(0<<COM0A0)|(1<<COM0B1)|(0<<COM0B0)|(1<<WGM01)|(1<<WGM00);

TCCR0B |= (1<<WGM02)|(1<<CS02)|(0<<CS01)|(0<<CS00);

while (1)

{

//Checks if the button is pressed while motor is off

while (!DCMotor)

{

if(!SWITCH)

{

_delay_ms(100);

ADCSRA |= ((1<<ADEN)|(1<<ADSC)|(0<<ADPS2)|(0<<ADPS1)|(1<<ADPS0));

ADMUX |= (1<<REFS0);

{

OCR0B = ADC;   //value being read from potentiometer,

sets duty cycle

DDRD = 0xFF;   //output

PORTD = 0x20;  //sets PD5 HIGH

DCMotor=1;	 //setting motor to 1 to get out of

while loop

}

}

}

//Checks if the button is pressed while motor is on

while (DCMotor)

{

if(!SWITCH)

{

_delay_ms(100);

// Turns off the motor and let it run CW

PORTD &= ~(1<<5);

DDRD = 0;

DCMotor=0;

}

}

}

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/2Bzs6oS3j8g

- GITHUB LINK OF THIS DA

https://github.com/acexhp/submission_da.git

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Allis Hierholzer