CPE301 – SPRING 2019

# Design Assignment 5

Student Name: Allis Hierholzer

Student #: 2000160356

Student Email: hierholz@unlv.nevada.edu

Primary Github address: https://github.com/acexhp/submission_da.git

Directory: Repository/cpe301/DesignAssignment/DA5

Task:

This DA is to be implemented in groups of two students. The goal of the assignment is to develop the above code to do the following:

1. Interface the provided NRF24L01+ RF Module to the ATmega328p using the SPI interface. Using the earlier developed code for ADC, transmit the ADC value of the internal temperature sensor, or LM35 sensor between two RF Modules. The ATmega328p interfacing the RF Modules should alternate between TX and RX modes every 0.5 secs (hopefully they are not both at TX and RX modes in the same interval). The temperature of both ATmega328p’s should be displayed on both ATmega328p’s.

Submission:

The following are required for successful completion of the design assignment:

a. AVR C code that has been compiled and working.

b. The C code should be well documented with explanation of every instruction.

c. A word document that contains the flow chart of the assembly code along with the snapshots of the schematics, components connected on the breadboard and screenshots.

d. Specify your assignment partner’s name on the submission doc.

Partner: Francisco Mata Carlos

- COMPONENTS LIST

- Breadboard

- Wires

- FTDI Chip

- nRF24L01 Single Chip 2.4 GHz Transceiver

- ATMEGA328P XPLAINED MINI

- ATMEL STUDIO 7.0

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* DA5.c

*/

//clock frequency

#ifndef F_CPU

#define F_CPU 16000000UL

#endif

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

#include <stdbool.h>

#include <stdio.h>

#include <string.h>

//Set up UART for printf();

#ifndef BAUD

#define BAUD 9600

#endif

#include "inc\STDIO_UART.c"

//Include nRF24L01+ library

#include "inc\nrf24l01.c"

#include "inc\nrf24l01-mnemonics.h"

#include "inc\spi.c"

void print_config(void);

//Used in IRQ ISR

volatile bool message_received = false;

volatile bool status = false;

void set_adc (void);

volatile unsigned char temperature_value[3];

volatile uint8_t adc_num;

int main(void)

{

set_adc();

//Set message to send (message < 32 characters)

char tx_message[32];			// Define string array

strcpy(tx_message,"Hello World!");	// String into array

//Initialize UART

uart_init();

//Initialize nRF24L01+ and print configuration info

nrf24_init();

print_config();

//Start listening to incoming messages

nrf24_start_listening();

strcpy(tx_message,"Hello World!");	//Copying string to array

nrf24_send_message(tx_message);

while (1)

{

if (message_received)

{

//Message received, print it

message_received = false;

printf("Received message: %s\n",nrf24_read_message());

//Send message as response

_delay_ms(500);

status = nrf24_send_message(temperature_value);

if (status == true) printf("Message sent successfully\n");

}

}

}

//Interrupt on IRQ pin

ISR(INT0_vect)

{

message_received = true;

}

void print_config(void)

{

uint8_t data;

printf("Startup successful\n\n nRF24L01+ configured as:\n");

printf("-------------------------------------------\n");

nrf24_read(CONFIG,&data,1);

printf("CONFIG		0x%x\n",data);

nrf24_read(EN_AA,&data,1);

printf("EN_AA			0x%x\n",data);

nrf24_read(EN_RXADDR,&data,1);

printf("EN_RXADDR		0x%x\n",data);

nrf24_read(SETUP_RETR,&data,1);

printf("SETUP_RETR		0x%x\n",data);

nrf24_read(RF_CH,&data,1);

printf("RF_CH			0x%x\n",data);

nrf24_read(RF_SETUP,&data,1);

printf("RF_SETUP		0x%x\n",data);

nrf24_read(STATUS,&data,1);

printf("STATUS		0x%x\n",data);

nrf24_read(FEATURE,&data,1);

printf("FEATURE		0x%x\n",data);

printf("-------------------------------------------\n\n");

}

//  Interrupt used to follow instructions below when conversion is done

ISR(ADC_vect)

{

volatile unsigned int i=0;		// From characters to string

char temperature[3];

adc_num = (ADCL >> 1); 		//Shifts the value right to one place

itoa(adc_num, temperature, 10);	//Int to string

while (i<3)				//Making string from characters of

temperature_value

{

temperature_value[i] = temperature[i];

i++;

}

}

void set_adc (void)

{

ADMUX = 0x40;		//ADC shifts right

ADCSRA = 0xEF;  	//ADC enable, start conversion, auto trig enable, interrupt

enable, set prescaler

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/x6FYeoPLdWY

- GITHUB LINK OF THIS DA

https://github.com/acexhp/submission_da.git

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Allis Hierholzer